GIS APPLICATIONS IN TELECOMMUNICATIONS

5-Day Intensive Training Program

DAY 4 LAB MANUAL

Fiber, Transmission & Microwave Link Planning

Version 3.0  |  Instructor-led Hands-on Manual

Platforms: ArcGIS Pro (primary) + ArcMap Geometric/Utility Network notes
Duration: 1 Day (09:00 – 16:00)
Scenario: MetroTel — Capital Region (continues Day 3)

Includes: Objectives · Agenda · Real & synthetic data · Geometric network · LOS/Profile · Viewshed · Checkpoints · Quiz

| Document | Detail |
| --- | --- |
| Course | GIS Applications in Telecommunications |
| Day | Day 4 — Fiber, Transmission & Microwave Link Planning |
| Version | 3.0 |
| Depends on | Day 1 (GDB design) + Day 2 (sites) + Day 3 (priority expansion zones) |
| Primary tools | Geometric Network / Utility Network Analyst (ArcMap), Feature-to-Network connectivity design, 3D Analyst Line of Sight, Profile Graph / Stack Profile, Viewshed / Geodesic Viewshed, Spatial Analyst |
| Data folder | Day4_Fiber_Microwave/data/ |

# 1. Learning Objectives

By the end of Day 4, participants will be able to:

Design FTTH/transmission node-edge topology (OLT → Joint → FDH → Splitter → ONT) in a geodatabase.

Build and use a geometric network (ArcMap Utility Network Analyst) — or Pro connectivity workflow — to trace fiber connectivity.

Map microwave hubs/remotes and evaluate Line of Sight (LOS) with 3D Analyst.

Extract and interpret terrain profiles (Profile Graph / Stack Profile) for Fresnel/clearance discussion.

Run Viewshed analysis for broader visibility/obstruction checks around microwave sites.

Recommend accept / redesign / raise-antenna decisions for planned links serving Day 3 expansion zones.

# 2. Day Agenda

| Time | Session | Outcome |
| --- | --- | --- |
| 09:00–09:30 | Recap Day 3 + transport concepts | Fiber vs microwave roles clear |
| 09:30–10:45 | Lab 1 — Nodes, fiber routes, GDB topology | FTTH/transmission layers ready |
| 10:45–11:00 | Break | — |
| 11:00–12:15 | Lab 2 — Geometric network & tracing | Connectivity / isolation traces work |
| 12:15–13:15 | Lunch | — |
| 13:15–14:30 | Lab 3 — Microwave LOS + profile graph | LOS pass/fail + profile interpreted |
| 14:30–14:45 | Break | — |
| 14:45–15:40 | Lab 4 — Viewshed & expansion decision | Visibility map + recommendations |
| 15:40–16:00 | Outputs, quiz, bridge to Day 5 | Day 4 deliverables complete |

# 3. Concepts Quick Brief

## 3.1 Why GIS for fiber & microwave?

Fiber is a connected network problem (what feeds what; where cuts isolate customers).

Microwave is a terrain/obstacle visibility problem (LOS, Fresnel, viewshed).

Both must align with commercial demand zones from Day 3.

## 3.2 Key terms

| Term | Meaning in this course |
| --- | --- |
| OLT | Optical Line Terminal — central GPON headend |
| FDH | Fiber Distribution Hub / cabinet |
| Splitter | Passive optical split (e.g., 1:8, 1:16) |
| ONT | Optical Network Terminal at customer/building |
| Feeder / Distribution | Fiber hierarchy: core feeders vs access distribution |
| Geometric Network | ArcMap network of junctions + edges with connectivity rules |
| Utility Network Analyst | ArcMap toolbar for trace, flow, flags on geometric networks |
| LOS | Line of Sight between TX and RX antennas |
| Profile Graph | Cross-section of elevation along a link |
| Viewshed | Cells visible from observer point(s) |
| Fresnel zone | Ellipsoid clearance volume around MW path (discussed conceptually) |

# 4. Datasets — Synthetic (lab) + Real (optional)

## 4.1 Synthetic data included

| File | Type | Content | Use |
| --- | --- | --- | --- |
| NetworkNodes.csv | Points table | OLT, FDH, Splitter, Joint, MW Hub/Remote | Network junctions |
| FiberRoutes.geojson | Lines | Feeder + distribution fiber (Existing/Planned) | Edges for fiber network |
| MicrowaveLinks.geojson | Lines | Existing / Planned / Candidate MW links | LOS & MW topology |
| FTTH_ONT_Buildings.csv | Points | 80 sample buildings/ONTs | Access endpoints |
| ObstructionPolygons.geojson | Polygons | Synthetic buildings/hill proxies with HEIGHT_M | LOS obstruction demo |
| SyntheticDEM_Points.csv | XYZ points | ~1,100 elevation samples | Build TIN/raster DEM offline |
| ViewshedObservers.csv | Points | MW observers + OFFSETA heights | Viewshed inputs |
| ConnectivityRules.csv | Table | Allowed node-to-node rules | Geometric network design |
| LOS_Workflow_Checklist.csv | Table | 6-step LOS procedure | Lab 3 guide |
| Day4_AllTables.xlsx | Excel | All tabular sheets | Instructor view |
| REAL_DATA_SOURCES.txt | Text | SRTM/OSM/HDX links | Optional real path |

## 4.2 Optional REAL data

DEM: SRTM 30 m / NASADEM / Copernicus GLO-30 / AW3D30 (preferred for LOS/viewshed).

Buildings: OSM footprints (Geofabrik); extrude if heights available.

Roads: OSM highways for trench routing context.

Day 3 MarketingZones / suitability ranks for “where to extend fiber/MW”.

NOTE: Synthetic DEM is for classroom continuity. For realistic microwave training, switch to real SRTM/NASADEM as soon as possible.

# 5. Software & Project Setup

## 5.1 Required extensions

ArcGIS Pro: 3D Analyst (LOS, Profile, Viewshed); Spatial Analyst helpful.

ArcMap (Lab 2 classic path): Geometric Network + Utility Network Analyst toolbar.

If ArcMap is unavailable: use Pro “functional connectivity” path in Lab 2B (attribute-based trace + topology).

## 5.2 Create Day 4 project

Open ArcGIS Pro → New Map project MetroTel_Day4_FiberMW.aprx

Folder connection to .../Day4_Fiber_Microwave/data

Create file GDB MetroTel_Day4.gdb

Optional: also open ArcMap 10.x mxd MetroTel_Day4_GeometricNetwork.mxd for Lab 2A

Set a projected CRS in meters (local UTM) for DEM/LOS work; keep WGS84 sources and Project them.

## 5.3 Load & convert baseline layers

XY Table To Point: NetworkNodes.csv → NetworkNodes (LON/LAT, GCS WGS84) → Project to UTM → NetworkNodes_UTM

Import FiberRoutes.geojson → FiberRoutes → Project to FiberRoutes_UTM

Import MicrowaveLinks.geojson → MicrowaveLinks → Project to MicrowaveLinks_UTM

XY Table To Point: FTTH_ONT_Buildings.csv → ONT_Buildings

Import ObstructionPolygons.geojson → Obstructions

XY Table To Point: ViewshedObservers.csv → ViewshedObservers

Symbolize nodes uniquely by NODE_TYPE; lines by STATUS (Existing/Planned/Candidate)

CHECKPOINT: Map shows OLTs/FDHs/joints, fiber feeders, MW links, ONTs, and obstruction polygons.

# 6. Lab 1 — Build Fiber/Transmission Feature Dataset

Objective: Organize hierarchical FTTH/transmission layers ready for network connectivity.

In MetroTel_Day4.gdb create Feature Dataset FD_Transport (set UTM spatial reference).

Move/copy NetworkNodes_UTM, FiberRoutes_UTM, ONT_Buildings (projected) into FD_Transport.

Add fields if missing: ENABLED (short, default 1), FROM_NODE, TO_NODE already on edges.

Create topology FD_Transport_Topo (optional but recommended):

FiberRoutes must not have dangles (except intentional ends).

Fiber endpoints must be covered by NetworkNodes.

Nodes must be covered by FiberRoutes endpoints (for network junctions).

Validate topology; fix obvious digitizing gaps by snapping planned routes to JH-04 / FDH-04 / FDH-06.

Attribute QA: select STATUS = 'Planned' fiber — confirm they serve Day 3 west/tech-park expansion story.

Create relationship-friendly IDs: ensure every FiberRoutes.FROM_NODE/TO_NODE exists in NetworkNodes.NODE_ID.

CHECKPOINT: Feature dataset + clean node/edge IDs; planned west fiber visible.

# 7. Lab 2 — Geometric Network & Connectivity Tracing

Objective: Establish and test connectivity between FTTH/transmission elements.

## 7.1 Lab 2A — ArcMap Geometric Network (curriculum path)

Open ArcMap → add FD_Transport layers (File GDB).

In Catalog, right-click FD_Transport → New → Geometric Network

Name: GN_FiberTransport

Select participating classes: NetworkNodes (junction), FiberRoutes (simple edge)

Snap sources/sinks optionally: set OLT nodes as Sources (for flow-style traces)

Use ConnectivityRules.csv to set weights/roles conceptually; configure connection rules so OLT–Joint–FDH–Splitter hierarchy is allowed

Build geometric network; add Utility Network Analyst toolbar

Set Analysis flags: Add Junction Flag on OLT-01

Add Edge Barrier on one feeder (e.g., FX-05) to simulate cut

Trace Tasks → Trace Downstream / Find Connected / Find Disconnected

Record which FDHs/ONTs become isolated when FX-07/FX-08 path is barred (south corridor)

Clear barriers; place flag on FDH-03; Find Path / Find Upstream back to OLT-01

TIP: If Utility Network Analyst is missing, customize toolbars → enable “Utility Network Analyst”.

## 7.2 Lab 2B — ArcGIS Pro functional connectivity path (if no ArcMap)

Use Select By Attributes / Select By Location to emulate traces:

Select FiberRoutes where FROM_NODE='OLT-01' OR TO_NODE='OLT-01' (first hop).

Iteratively select nodes touching selected edges, then edges touching those nodes (Expand selection).

Build a “TraceLog” table manually for classroom: hop number, NODE_ID, ROUTE_ID.

Simulate outage: remove/disable FX-03 and re-run selection expansion from OLT-01 — note OLT-02/FDH-02 isolation risk if no alternate path.

Optional Pro: create Trace Network (if license/version supports) on FD_Transport with same junction/edge roles.

NOTE: Curriculum emphasizes ArcMap geometric network; Pro Trace Network is the modern equivalent where available.

CHECKPOINT: You can demonstrate at least one connected trace and one isolation/outage scenario.

# 8. Lab 3 — Microwave LOS & Terrain Profile

Objective: Evaluate microwave links with Line of Sight and profile tools.

## 8.1 Build DEM

Path A (offline): Create TIN or raster from SyntheticDEM_Points.csv

XY Table To Point → DEM_Points; Project to UTM

Create TIN (3D Analyst) using ELEV_M, or Topo to Raster / IDW → DEM_Synth

Path B (real data): Load SRTM/NASADEM clipped to Capital Region → DEM_Real

Set DEM as ground surface in scene or keep as raster for geoprocessing

## 8.2 Line of Sight (existing vs planned)

Select MicrowaveLinks where LINK_ID in ('MWL-01','MWL-05','MWL-07') — one good, one planned over hills, one candidate

3D Analyst Tools → Visibility → Line Of Sight

Input Surface = DEM_Synth (or DEM_Real)

Target / Observer features: use MicrowaveLinks as 3D lines OR run LOS between NetworkNodes MW points

Observer offset = TX_HEIGHT_M / OFFSETA; Target offset = RX_HEIGHT_M / OFFSETB (from link attributes / ViewshedObservers)

Output = LOS_Results with Visible / Not Visible segments

Symbolize visible = green, obstructed = red

Overlay Obstructions polygons; discuss whether buildings/hills cause failures on MWL-05 (Central→West Planned)

## 8.3 Profile Graph

Convert/ensure MW link lines are 3D using Interpolate Shape against DEM → MWLinks_3D

In Pro: Analysis chart / Stack Profile OR 3D Explorer profile; in ArcMap: Profile Graph tool

Create profiles for MWL-01 (expect clearer) and MWL-05 (expect terrain bulge near west hill)

Mark approximate first Fresnel clearance conceptually (0.6 Fresnel discussed; not full RF engineering)

Decision log in a table Link_Decision.csv fields: LINK_ID, LOS_STATUS, OBSTRUCTION_NOTE, ACTION (Accept / Raise antenna / Reroute / Prefer fiber)

CHECKPOINT: LOS colors show pass/fail; profile exported/screenshot for MWL-05; decision noted.

# 9. Lab 4 — Viewshed & Expansion Recommendation

Objective: Broader visibility/obstruction check and final transport recommendation.

Viewshed / Geodesic Viewshed tool

Input raster = DEM; Observer features = ViewshedObservers (or MW hubs only)

Surface/Observer offset: use OFFSETA_M (antenna heights)

Optional: outer radius 8–15 km; inner radius 0

Output = Viewshed_MW (binary or frequency if multiple observers)

Zonal comparison: which Day 3 high-suitability / planned FDH areas fall inside MW-01 viewshed?

Combine results:

If west expansion (FDH-04 / MW-05) fails LOS and viewshed is weak → prefer planned fiber FX-09/FX-10

If airport/north remains visible → keep MW for rapid backhaul

Candidate MWL-07: accept only if LOS clear; else drop

Layout 1: Fiber geometric network + OLTs/FDHs + outage example

Layout 2: MW links with LOS green/red + DEM hillshade

Layout 3: Viewshed + planned expansion nodes from Day 3/4

Export Day4_FiberMW_Layouts.pdf and short recommendation paragraph

CHECKPOINT: Viewshed created; written recommendation for west expansion transport mode.

# 10. Expected Outputs Checklist

| # | Deliverable | Done? |
| --- | --- | --- |
| 1 | MetroTel_Day4_FiberMW.aprx + MetroTel_Day4.gdb |  |
| 2 | FD_Transport with nodes + fiber routes (+ topology optional) |  |
| 3 | Geometric network (ArcMap) OR documented Pro connectivity trace |  |
| 4 | At least one outage/isolation trace result |  |
| 5 | DEM (synthetic or real) used in analysis |  |
| 6 | LOS results for ≥2 microwave links |  |
| 7 | Profile graph/screenshot for obstructed/planned link |  |
| 8 | Viewshed from MW observer(s) |  |
| 9 | Recommendation: fiber vs MW for west/tech-park expansion |  |
| 10 | Exported layout PDF |  |

# 11. Instructor Answer Key — Typical Findings

Existing MW hub links (Central–North/East/South) often pass or mostly pass on synthetic DEM.

Planned Central–West (MWL-05) is designed to interact with the west hill/obstructions — expect partial/fail LOS → teach redesign.

Fiber planned branches FX-09/FX-10/FX-11 are the fallback to serve FDH-04 and FDH-06.

Cutting a south feeder can isolate FDH-05 / SPL-03 in simple tree topology — good outage demo.

Viewshed from MW-01 should cover central/north better than behind western ridge.

# 12. Troubleshooting

| Problem | Likely cause | Fix |
| --- | --- | --- |
| Geometric Network wizard missing | Using Pro only / wrong GDB type | Use ArcMap on File GDB feature dataset; or Lab 2B |
| LOS all not visible | Offsets=0 or DEM units mismatch | Set antenna offsets; DEM in meters; projected CRS |
| Profile flat/wrong | Line not interpolated to surface | Run Interpolate Shape first |
| Viewshed empty | Observers outside DEM / wrong offsets | Clip DEM larger; check OFFSETA_M |
| Nodes not connecting | Gaps / units / no snap | Snap fiber ends to nodes; rebuild network |
| Cannot find Utility Network Analyst | Toolbar not enabled | ArcMap Customize → Toolbars |

# 13. Quick Quiz

Name the typical FTTH hierarchy from core to customer.

What ArcMap toolbar traces a geometric network?

What is the difference between Line of Sight and Viewshed?

Why might a planned MW link fail even if endpoints are “close” on a 2D map?

If LOS fails to a Day 3 high-value expansion zone, what alternative does Day 4 data provide?

Suggested answers:

1) OLT → (Joint) → FDH → Splitter → ONT/Building.

2) Utility Network Analyst.

3) LOS = visibility along one path between points; Viewshed = area visible from observer(s).

4) Terrain/buildings intrude into the path/Fresnel zone; antenna heights too low.

5) Planned fiber feeders (FX-09/10/11) to FDH-04/FDH-06.

# 14. Bridge to Day 5

Day 5 packages coverage (Day 2), geo-marketing (Day 3), and fiber/microwave (Day 4) into ArcGIS Online web maps, dashboards, and Story Maps for executive reporting. Keep today’s LOS results, viewshed, and fiber trace maps ready to publish.

# 15. Version History

| Version | Changes |
| --- | --- |
| v1 | Draft Day 4 fiber/MW outline |
| v2 | Added LOS/viewshed lab sequence |
| v3 | Full manual with MetroTel synthetic fiber+MW datasets, DEM points, obstruction polygons, ArcMap geometric network steps, Pro fallback, LOS/profile/viewshed labs, checkpoints, quiz, real-data options |

— End of Day 4 Manual (Version 3.0) —