# Day 4 — Fiber Transmission Geometric Network Dataset

ArcGIS Desktop (ArcMap) training package: MetroTel Capital Region FTTH geometric network with multi-source internet distributed to more than 20,000 end-users.

# Dataset summary

project: MetroTel Capital Region - FTTH Geometric Network

software_target: ArcGIS Desktop (ArcMap) Geometric Network

crs_shapefiles: EPSG:32642

junction_count: 22889

edge_count: 22893

home_ont_count: 21420

internet_sources: 3

topology: Source -> Core -> Hub -> OLT -> Joint -> FDH -> Splitter -> Home_ONT

# Junction (node) inventory

| JuncType | Count |
| --- | --- |
| Home_ONT | 21420 |
| Optical_Splitter | 1260 |
| FDH | 90 |
| Joint_Closure | 90 |
| OLT | 18 |
| Regional_Hub | 6 |
| Internet_Source | 3 |
| Core_POP | 2 |

# Edge inventory

| EdgeType | Count |
| --- | --- |
| Drop_Fiber | 21420 |
| Distribution_Fiber | 1260 |
| Feeder_Fiber | 198 |
| Backbone_Fiber | 9 |
| Hub_Ring | 5 |
| Core_Ring | 1 |

# Geometric network structure

Internet_Source → Core_POP → Regional_Hub → OLT → Joint_Closure → FDH → Optical_Splitter → Home_ONT

Sources (AncillaryR=1): Internet_Source. Sinks (AncillaryR=2): Home_ONT. Intermediate devices: AncillaryR=0.

# How to build in ArcMap

Create File GDB MetroTel_FTTH_GN.gdb and Feature Dataset FD_FTTH (EPSG:32642).

Import shapefiles/GN_Junctions.shp and shapefiles/GN_Edges.shp into FD_FTTH.

Right-click FD_FTTH → New → Geometric Network → name GN_MetroTel_FTTH.

Set GN_Junctions as Simple Junction and GN_Edges as Simple Edge; snap tolerance 0.5 m.

Set Ancillary Roles: Internet_Source = Source, Home_ONT = Sink.

Open Utility Network Analyst and run Trace Upstream / Trace Downstream exercises.

Optional: run scripts/build_geometric_network_arcmap.py in ArcGIS Desktop Python.

# Package files

See README.md and docs/ArcMap_GeometricNetwork_Build_Guide.md for full file list, connectivity rules, and trace exercises.