GIS APPLICATION IN TELECOM

5-Day Intensive Training Program

DAY 1 LAB MANUAL

GIS concepts, coordinate systems, ArcGIS Pro, geodatabase, domains, subtypes, relationship classes, raster / DEM

Version 4.0  |  Follows presentation Day 1_ready.pptx  |  ArcGIS Pro  |  09:00–16:00

How to use: instructor shows a slide block; class does the matching lab in ArcGIS Pro. No sample CSV or GeoJSON files are required. You create empty shapefiles and geodatabase objects, type the key fields, digitize a few features by clicking, then build relationship classes.

# How this manual maps to the presentation

| Presentation | Slides | Do this in the manual |
| --- | --- | --- |
| Title, 5-day agenda, Day 1 overview, agenda, outcomes | 1–6 | Read only |
| Part 1 — GIS concepts and data types | 7–16 | Lab 1 — open Pro, map + table |
| Part 2 — Coordinate systems | 17–24 | Lab 2 — set the map CRS |
| Part 3 — Pro, GDB, shapefile, feature class, feature dataset | 25–31 | Lab 3 |
| Domains and subtypes | 32–34 | Lab 4 |
| Relationship classes, ERD, cardinality, irrigation | 35–41 | Lab 5 |
| Part 4 — Raster analysis and DEM | 42–50 | Lab 6 |
| Instructor corrections (yellow) | 51 | Delete before class |

# 1. Learning outcomes

Match presentation: Day 1 Learning Outcomes

Explain GIS as a system (data, software, hardware, people, methods).

Choose vector or raster for a given question (assets vs coverage / terrain).

Distinguish GCS vs projected CRS, and explain datum and WGS84.

Create a shapefile, a file geodatabase, a feature dataset, and feature classes in ArcGIS Pro.

Create a domain and subtypes, and assign them to fields.

Put a primary key on the parent and a foreign key on the child, then create a 1:M relationship class.

Add a DEM and derive slope, aspect, and hillshade.

# 2. Day agenda

| Time | Presentation block | Lab |
| --- | --- | --- |
| 09:00–09:30 | Slides 1–6 — program, 5-day agenda, Day 1 agenda & outcomes | — |
| 09:30–10:30 | Part 1 — GIS, keywords, functions, components, vector/raster | Lab 1 |
| 10:30–10:45 | Break | — |
| 10:45–12:00 | Part 2 — Earth, datum, WGS84, GCS vs PCS | Lab 2 |
| 12:00–13:00 | Lunch | — |
| 13:00–14:15 | Part 3 — Pro, geodatabase, shapefile, FC, feature dataset | Lab 3 |
| 14:15–14:50 | Domains and subtypes | Lab 4 |
| 14:50–15:00 | Break | — |
| 15:00–15:40 | ERD, cardinality, irrigation + telecom 1:M | Lab 5 |
| 15:40–16:00 | Raster / DEM / Let’s explore it in ArcGIS Pro | Lab 6 |

# 3. One-time project setup

Do this once before Lab 1. All later labs use the same project.

Open ArcGIS Pro.

Click New → Map.

Name the project Day1_GIS_Telecom. Save it in a local folder (for example C:\GIS_Training\Day1).

Note four places: Catalog pane, Contents pane, Map view, Ribbon (Map, Edit, Analysis, View).

Catalog → right-click Folders → Add Folder Connection → select your Day 1 folder.

Catalog → right-click Databases → New File Geodatabase → name it Day1.gdb.

Map tab → Basemap → Imagery or Topographic.

CHECKPOINT: Named project, folder connection, empty Day1.gdb.

# 4. Part 1 — GIS concepts and data types

Presentation: Part 1 · What is GIS through Advantages and Disadvantages of Raster

## 4.1 What to take from the slides

| Slide idea | Remember this |
| --- | --- |
| What is GIS | A GIS captures, stores, analyzes, and presents geographically referenced data. Software alone is not a GIS. |
| Keywords | Geographic = location. Information = attributes. Systems = hardware, software, data, procedures. |
| Functions | Acquire, store, manage, analyze, visualize. |
| Components | Hardware, software (ArcGIS Pro), data, people, methods. |
| Vector | Points, lines, polygons. Use for towers, fiber, parcels, homes. |
| Raster | Grid of cells. Use for elevation, imagery, rainfall, coverage. |
| Attributes / metadata | Attributes = the “what”. Metadata = source, date, CRS, accuracy. |

## 4.2 Lab 1 — See map and table together

Map tab: use Explore to pan and zoom.

In Catalog, expand Day1.gdb (still empty). Part 3 objects will live here.

Contents: turn the basemap on/off. That is a raster-style backdrop; your assets will be vector.

View → Catalog View, then return to the map.

TIP: After Lab 3, right-click a feature class → Attribute Table. Click a row → it highlights on the map.

CHECKPOINT: Student can name Catalog, Contents, Map, and Attribute Table.

# 5. Part 2 — Coordinate systems

Presentation: Part 2 · Earth, datum, WGS84, GCS, Projected Coordinate System

Earth is not a perfect sphere. Smooth model = ellipsoid; gravity surface = geoid.

A datum is the model of Earth that coordinates are measured from.

WGS84 is the usual global datum (GPS, EPSG:4326, degrees).

GCS = latitude/longitude. PCS = easting/northing in meters (example: WGS 84 / UTM zone 42N).

Define Projection only labels correct coordinates. Project converts to another CRS.

## 5.2 Lab 2 — Set the map to the presentation CRS

Contents → right-click the map name → Properties → Coordinate Systems.

Search WGS 1984 UTM Zone 42N (or 41N / 43N if that is your city).

Select it → OK. Pointer values should look like meters, not 34.5 / 69.2 degrees.

Map tab → Measure a short street. Units should be meters.

CHECKPOINT: GCS = degrees; PCS = meters; our map uses UTM.

# 6. Part 3 — Shapefile, geodatabase, feature dataset, feature classes

Presentation: ArcGIS Pro → Managing Vector Data → Geodatabase → Types of Geodatabases → Other GIS Data Types → Feature Dataset

Shapefile: .shp + .dbf + .shx + .prj. Cannot store domains, subtypes, or relationship classes.

File geodatabase (.gdb): the container for this course.

Feature class: one geometry type, same fields, same spatial reference.

Feature dataset: container inside the GDB; all classes share one CRS.

## 6.2 Data you will create (no files to import)

Irrigation (presentation): Scheme → Canal → Command Area → Structure (all 1:M).

Telecom parallel: OLT → FDH → Splitter → Home (all 1:M). Tower for subtypes.

| Model | Feature class | Geometry | PK | FK on child |
| --- | --- | --- | --- | --- |
| Irrigation | Scheme | Point | scheme_id | — |
| Irrigation | Canal | Polyline | canal_id | scheme_id |
| Irrigation | Command_Area | Polygon | area_id | canal_id |
| Irrigation | Structure | Point | structure_id | area_id |
| Telecom | OLT | Point | olt_id | — |
| Telecom | FDH | Point | fdh_id | olt_id |
| Telecom | Splitter | Point | splitter_id | fdh_id |
| Telecom | Home | Point | home_id | splitter_id |
| Wireless | Tower | Point | tower_id | — |

## 6.3 Lab 3A — Create a shapefile

Catalog → folder connection (not the .gdb) → New → Shapefile.

Name: Towers_shp. Geometry: Point. CRS: WGS 1984 UTM Zone 42N.

Design → Fields → add tower_id (Text, 20). Save.

On disk you should see .shp, .shx, .dbf, .prj.

NOTE: Do not put domains, subtypes, or relationship classes on the shapefile. Those belong in the geodatabase.

## 6.4 Lab 3B — Create two feature datasets

Right-click Day1.gdb → New → Feature Dataset → name FD_Irrigation → CRS WGS 1984 UTM Zone 42N → Finish.

Repeat: FD_Telecom, same CRS.

## 6.5 Lab 3C — Irrigation feature classes

Right-click FD_Irrigation → New → Feature Class. Repeat for each row.

Example Scheme: Name Scheme, geometry Point. Fields: scheme_id (Text 20, PK), scheme_name (Text 50), status (Text 20).

Canal (Polyline): canal_id (PK), scheme_id (FK), canal_name, status.

Command_Area (Polygon): area_id (PK), canal_id (FK), area_name, status.

Structure (Point): structure_id (PK), area_id (FK), struct_type, status.

## 6.6 Lab 3D — Telecom feature classes

Same wizard inside FD_Telecom.

OLT (Point): olt_id (PK), olt_name, status.

FDH (Point): fdh_id (PK), olt_id (FK), fdh_name, status.

Splitter (Point): splitter_id (PK), fdh_id (FK), split_ratio, status.

Home (Point): home_id (PK), splitter_id (FK), address, status.

Tower (Point): tower_id (PK), TowerType (Short integer — subtype field), height_m (Double), status.

CHECKPOINT: Nine feature classes. Each child has a foreign-key field named like the parent’s ID.

## 6.7 Lab 3E — Digitize a few features

No spreadsheet. Click a handful of features so keys have values.

Drag Scheme, Canal, Command_Area, Structure onto the map. Edit tab → Create.

One Scheme point: scheme_id = SCH-01, name North Scheme, status Active. Save.

Two Canal lines, both scheme_id = SCH-01, canal_id CAN-01 and CAN-02.

Two Command_Area polygons with canal_id CAN-01 / CAN-02, area_id CA-01 / CA-02.

Three Structure points inside the polygons; area_id matches the polygon; types intake, super passage, weir.

Telecom: 1 OLT (OLT-01), 2 FDHs with olt_id=OLT-01, 2 splitters with matching fdh_id, 3 homes with matching splitter_id.

Three towers T-01, T-02, T-03. TowerType can wait until Lab 4.

TIP: Home.splitter_id must equal an existing Splitter.splitter_id or the relationship class will not find children.

# 7. Lab 4 — Domains and subtypes

Presentation: Domains and Subtypes · Subtypes · example screenshot

## 7.2 Coded-value domain (status list)

Right-click Day1.gdb → Domains → New.

Name Dom_Status. Field Type Text. Domain Type Coded Value.

Codes: Active / Active; Planned / Planned; OnAir / On Air. Save.

## 7.3 Range domain (tower height)

New domain Dom_Height_m. Field Type Double. Domain Type Range. Minimum 3, Maximum 80. Save.

## 7.4 Assign domains to fields

Tower → Design → Fields. status → Domain Dom_Status. height_m → Dom_Height_m. Save.

Assign Dom_Status to the status field on Scheme, Canal, Command_Area, Structure, OLT, FDH, Splitter, Home.

## 7.5 Subtypes on Tower

Tower → Design → Subtypes. Subtype Field: TowerType.

Code 1 Macro tower; 2 Small cell; 3 Rooftop antenna (same as the presentation).

Optional: different default height_m per subtype (e.g. 30 / 8 / 12). Save.

CHECKPOINT: Status is a list; height rejects values outside 3–80; Tower has three subtypes.

# 8. Lab 5 — Keys and relationship classes

Presentation: Relationship Classes · ERD · Cardinality · Irrigation example

Origin holds the primary key. Destination holds the matching key.

1:M is the usual pattern. Composite deletes children with the parent; use Simple unless you want that.

| Name | Origin | Origin PK | Destination | Dest FK | Card. |
| --- | --- | --- | --- | --- | --- |
| RC_Scheme_Canal | Scheme | scheme_id | Canal | scheme_id | 1:M |
| RC_Canal_CommandArea | Canal | canal_id | Command_Area | canal_id | 1:M |
| RC_CommandArea_Structure | Command_Area | area_id | Structure | area_id | 1:M |
| RC_OLT_FDH | OLT | olt_id | FDH | olt_id | 1:M |
| RC_FDH_Splitter | FDH | fdh_id | Splitter | fdh_id | 1:M |
| RC_Splitter_Home | Splitter | splitter_id | Home | splitter_id | 1:M |

## 8.3 Create one relationship class (full clicks) — then repeat

Right-click FD_Irrigation → New → Relationship Class.

Origin: Scheme. Destination: Canal. Name: RC_Scheme_Canal.

Type: Simple. Cardinality: One to many (1:M). No relationship attributes.

Origin primary key: scheme_id. Destination foreign key: scheme_id on Canal.

Forward label: feeds canals. Backward: belongs to scheme. Finish.

Repeat the same wizard for the other five rows. Only names and key fields change.

## 8.5 Prove it

Select the scheme point. Open related records / Select Related Records — both canals with scheme_id SCH-01 should appear.

Select one splitter; related homes should be the houses with that splitter_id.

CHECKPOINT: Six relationship classes. Selecting a parent lists its children. FK is stored on the child.

NOTE: Tower has no child class here. Subtypes split towers in one feature class. Tower → Sector later: add a Sector table with tower_id and create RC_Tower_Sector the same way.

# 9. Lab 6 — Raster analysis and DEM

Presentation: Part 4 · Weighted Overlay · Resolution · DEM · Let’s Explore It in ArcGIS Pro

Do not build a fake elevation table. Use Living Atlas / a local SRTM GeoTIFF.

Add a DEM: Portal → Living Atlas → search Terrain/DEM, or add a local GeoTIFF.

Properties → Source: write down cell size (resolution) and CRS. Project to UTM if it is still in degrees.

Analysis → Tools → Hillshade (do this first).

Run Slope (Spatial Analyst). Optional: Aspect, Contour (20 m), Viewshed from one Tower (offset 30 m).

Optional Weighted Overlay: reclassify slope 1–5 as a one-layer demo. Do not invent extra criteria tables.

CHECKPOINT: DEM on the map; Hillshade and Slope created; student can state cell size.

# 10. Expected outputs

| # | Deliverable | Done? |
| --- | --- | --- |
| 1 | Day1_GIS_Telecom.aprx and Day1.gdb |  |
| 2 | Map CRS = WGS 1984 UTM (your zone) |  |
| 3 | Shapefile Towers_shp (comparison only) |  |
| 4 | FD_Irrigation and FD_Telecom |  |
| 5 | All feature classes with PK and FK fields |  |
| 6 | Domains Dom_Status and Dom_Height_m assigned |  |
| 7 | Subtypes on Tower (Macro / Small cell / Rooftop) |  |
| 8 | Six 1:M relationship classes; related records work |  |
| 9 | DEM + Hillshade + Slope |  |

# 11. Quick quiz

Name the five GIS components from the slide.

Vector or raster for a fiber cable? For a DEM?

What is the difference between a datum and a projected coordinate system?

Why can a shapefile not hold a relationship class?

Where is the foreign key stored in Splitter → Home?

What integer codes did we use for Tower subtypes?

Name three rasters you can derive from a DEM.

Answers: 1) Hardware, software, data, people, methods. 2) Fiber = vector line; DEM = raster. 3) Datum = Earth model; PCS = flat meter grid. 4) Shapefiles have no geodatabase behavior. 5) Home.splitter_id. 6) 1 Macro, 2 Small cell, 3 Rooftop. 7) Slope, aspect, hillshade, contour, viewshed, watershed, or profile.

# 12. Instructor notes

Teach from Day 1_ready.pptx. After each part-divider, switch to the matching lab.

Delete presentation slide 51 (yellow correction log) before class.

Do not distribute the old Day 1 CSV/GeoJSON pack for this version.

# 13. Version history

| Version | Changes |
| --- | --- |
| v3 | MetroTel synthetic datasets and concept-heavy labs (previous) |
| v4 | Aligned to Day 1_ready.pptx. No synthetic tables. Click-by-click shapefile, FD, FC, domains, subtypes, keys, relationship classes, DEM. |

— End of Day 1 Manual (Version 4.0) —